Bảng API chính

| Module | API | Chức năng | Khai báo | Định nghĩa |
| --- | --- | --- | --- | --- |
| App tasks | app_tasks_init() | Khởi tạo UART, RTC, ADC, DAC, TX/RX | app_tasks.h (line 7) | app_tasks.c (line 878) |
| App tasks | app_tasks_start() | Start DAC/ADC/TX/RX/TIM2 theo board role | app_tasks.h (line 15) | app_tasks.c (line 904) |
| App tasks | app_tasks_process() | Main-loop task, UART command, logs, RX queue | app_tasks.h (line 17) | app_tasks.c (line 947) |
| App tasks | app_tasks_on_timer_period_elapsed() | TIM2 ISR dispatch theo board role | app_tasks.h (line 19) | app_tasks.c (line 927) |
| Protocol | app_protocol_get_expected_payload() | Lấy payload chuẩn 55 A5 3C C3 | app_protocol.h (line 14) | app_protocol.c (line 6) |
| Protocol | app_protocol_checksum() | Tính checksum LEN + sum(payload) | app_protocol.h (line 17) | app_protocol.c (line 17) |
| Protocol | app_protocol_get_expected_frame() | Build frame chuẩn để RX so sánh | app_protocol.h (line 19) | app_protocol.c (line 31) |
| Protocol | app_protocol_build_frame() | Build frame AA AA AA D5 LEN PAYLOAD CHECKSUM | app_protocol.h (line 21) | app_protocol.c (line 39) |
| TX | vlc_tx_init() | Init TX frame + TIM1 handle | vlc_tx.h (line 8) | vlc_tx.c (line 28) |
| TX | vlc_tx_start() | Start TIM1 PWM carrier | vlc_tx.h (line 10) | vlc_tx.c (line 43) |
| TX | vlc_tx_on_bit_tick() | Phát 1 bit OOK mỗi TIM2 tick | vlc_tx.h (line 12) | vlc_tx.c (line 55) |
| TX | vlc_tx_set_payload() | Đổi payload runtime, rebuild TX frame | vlc_tx.h (line 14) | vlc_tx.c (line 102) |
| TX | vlc_tx_set_enabled() | Bật/tắt truyền liên tục | vlc_tx.h (line 16) | vlc_tx.c (line 131) |
| TX | vlc_tx_set_carrier_test() | Bật/tắt carrier test mode | vlc_tx.h (line 18) | vlc_tx.c (line 147) |
| TX | vlc_tx_send_single_frame() | Gửi 1 frame rồi stop | vlc_tx.h (line 20) | vlc_tx.c (line 160) |
| TX | vlc_tx_get_frame_count() | Số frame TX đã hoàn tất | vlc_tx.h (line 30) | vlc_tx.c (line 195) |
| TX | vlc_tx_get_frame_snapshot() | Copy frame TX hiện tại cho log/status | vlc_tx.h (line 32) | vlc_tx.c (line 201) |
| RX | vlc_rx_init() | Init RX queue, stats, FSM | vlc_rx.h (line 42) | vlc_rx.c (line 67) |
| RX | vlc_rx_start() | Start TIM4 edge counter | vlc_rx.h (line 44) | vlc_rx.c (line 90) |
| RX | vlc_rx_on_bit_tick() | Đọc TIM4 delta, tạo raw bit, gọi FSM | vlc_rx.h (line 46) | vlc_rx.c (line 105) |
| RX | vlc_rx_pop_frame() | Pop valid RX frame + frame id | vlc_rx.h (line 50) | vlc_rx.c (line 216) |
| RX | vlc_rx_get_stats() | Lấy RX counters/debug values | vlc_rx.h (line 52) | vlc_rx.c (line 231) |
| RX Sync | rx_sync_init() | Reset FSM đồng bộ frame | rx_sync.h (line 42) | rx_sync.c (line 60) |
| RX Sync | rx_sync_on_tick() | FSM xử lý 1 raw bit | rx_sync.h (line 44) | rx_sync.c (line 73) |
| RX Sync | rx_sync_get_state() | Lấy state hiện tại của FSM | rx_sync.h (line 46) | rx_sync.c (line 66) |
| UART | bsp_uart_init() | Bind UART handle | bsp_uart.h (line 9) | bsp_uart.c (line 7) |
| UART | bsp_uart_printf() | In log UART blocking | bsp_uart.h (line 11) | bsp_uart.c (line 13) |
| UART | bsp_uart_read_byte() | Poll 1 byte command UART | bsp_uart.h (line 13) | bsp_uart.c (line 42) |
| ADC | bsp_adc_monitor_init() | Init ADC monitor | bsp_adc_monitor.h (line 21) | bsp_adc_monitor.c (line 15) |
| ADC | bsp_adc_monitor_start() | Start ADC DMA/monitor | bsp_adc_monitor.h (line 23) | bsp_adc_monitor.c (line 21) |
| ADC | bsp_adc_monitor_get_voltages() | Lấy điện áp đã scale mV | bsp_adc_monitor.h (line 27) | bsp_adc_monitor.c (line 59) |
| DAC | bsp_dac_threshold_init() | Init DAC threshold | bsp_dac_threshold.h (line 8) | bsp_dac_threshold.c (line 8) |
| DAC | bsp_dac_threshold_set_mv() | Set DAC threshold mV | bsp_dac_threshold.h (line 10) | bsp_dac_threshold.c (line 15) |
| DAC | bsp_dac_threshold_get_mv() | Đọc threshold hiện tại | bsp_dac_threshold.h (line 12) | bsp_dac_threshold.c (line 37) |
| RTC | bsp_rtc_init() | Init LSE RTC + backup magic | bsp_rtc.h (line 9) | bsp_rtc.c (line 173) |
| RTC | bsp_rtc_set_datetime() | Set RTC date/time | bsp_rtc.h (line 14) | bsp_rtc.c (line 124) |
| RTC | bsp_rtc_get_datetime_string() | Format YYYY-MM-DD HH:MM:SS | bsp_rtc.h (line 12) | bsp_rtc.c (line 227) |
| RTC | bsp_rtc_print_log() | In dòng rtc ... | bsp_rtc.h (line 13) | bsp_rtc.c (line 259) |
| RTC | bsp_rtc_print_backup_log() | In dòng rtc_bkp ... | bsp_rtc.h (line 17) | bsp_rtc.c (line 272) |

Bảng define cấu hình chính

| Nhóm | Define | Giá trị | Vị trí |
| --- | --- | --- | --- |
| Bit rate | APP_BIT_RATE_HZ | 100000U | app_config.h (line 6) |
| TIM2 | APP_TIM2_TICK_HZ | 100000U | app_config.h (line 7) |
| TIM2 | APP_TIM2_ARR_100KHZ | 839U | app_config.h (line 8) |
| TIM1 PWM | APP_TIM1_PWM_PERIOD | 167U | app_config.h (line 10) |
| OOK | APP_TIM1_CARRIER_CCR_ON | 84U | app_config.h (line 11) |
| OOK | APP_TIM1_CARRIER_CCR_OFF | 0U | app_config.h (line 12) |
| RX | APP_RX_EDGE_THRESHOLD | 6U | app_config.h (line 15) |
| Frame | APP_FRAME_PREAMBLE_BYTE | 0xAAU | app_config.h (line 18) |
| Frame | APP_FRAME_PREAMBLE_BYTES | 3U | app_config.h (line 19) |
| Frame | APP_FRAME_SYNC_BYTE | 0xD5U | app_config.h (line 20) |
| Frame | APP_MAX_PAYLOAD_LEN | 32U | app_config.h (line 21) |
| Queue | APP_RX_QUEUE_DEPTH | 32U | app_config.h (line 22) |
| Queue | APP_RX_ERROR_EVENT_QUEUE_DEPTH | 16U | app_config.h (line 23) |
| Role | APP_BOARD_ROLE_TX_ONLY | 0U | app_config.h (line 25) |
| Role | APP_BOARD_ROLE_RX_ONLY | 1U | app_config.h (line 26) |
| Role | APP_BOARD_ROLE_TX_RX_LOOPBACK | 2U | app_config.h (line 27) |
| Role | APP_BOARD_ROLE | APP_BOARD_ROLE_TX_ONLY | app_config.h (line 29) |
| DAC | APP_DAC_DEFAULT_THRESHOLD_MV | 1650U | app_config.h (line 35) |
| ADC | APP_ADC_CHANNEL_COUNT | 7U | app_config.h (line 37) |
| ADC | APP_ADC_VREF_MV | 3300U | app_config.h (line 38) |
| Log | APP_LOG_PERIOD_MS | 1000U | app_config.h (line 40) |
| Error log | APP_ERR_LOG_MAX_PER_PERIOD | 5U | app_config.h (line 41) |
| Error bits | APP_ERR_BITS_MAX_PER_PERIOD | 1U | app_config.h (line 42) |
| RTC | APP_RTC_ENABLE | 1U | app_config.h (line 45) |
| RTC | APP_RTC_USE_LSE | 1U | app_config.h (line 46) |
| RTC | APP_RTC_MAGIC | 0x32F4U | app_config.h (line 47) |
| RTC | APP_RTC_DEFAULT_YEAR | 2026U | app_config.h (line 48) |
| RTC | APP_RTC_LOG_ENABLE | 1U | app_config.h (line 54) |
| RX sync error | RX_SYNC_ERROR_SYNC | 1U | rx_sync.h (line 36) |
| RX sync error | RX_SYNC_ERROR_LENGTH | 2U | rx_sync.h (line 37) |
| RX sync error | RX_SYNC_ERROR_PREAMBLE | 3U | rx_sync.h (line 38) |
| Board pin | PIN_RX_OUT_D_Pin | GPIO_PIN_6 | board_pins.h (line 7) |
| Board pin | PIN_TX_READY_Pin | GPIO_PIN_13 | board_pins.h (line 10) |
| Board pin | PIN_RX_READY_Pin | GPIO_PIN_15 | board_pins.h (line 13) |
| HAL | HAL_RTC_MODULE_ENABLED | enabled | stm32f4xx_hal_conf.h (line 61) |
| HAL | LSE_VALUE | 32768U | stm32f4xx_hal_conf.h (line 127) |
| HAL | LSE_STARTUP_TIMEOUT | 5000U | stm32f4xx_hal_conf.h (line 131) |

Struct/Enum quan trọng

| Tên | Loại | Ý nghĩa | Vị trí |
| --- | --- | --- | --- |
| app_frame_t | struct | Frame decoded: length, payload, checksum | app_protocol.h (line 7) |
| rx_sync_state_t | enum | Các state FSM RX sync | rx_sync.h (line 7) |
| rx_sync_result_t | struct | Kết quả mỗi tick RX FSM | rx_sync.h (line 18) |
| vlc_rx_stats_t | struct | RX counters/debug stats | vlc_rx.h (line 8) |
| vlc_rx_error_event_t | struct | Deferred RX error event | vlc_rx.h (line 23) |
| bsp_adc_voltages_t | struct | Các điện áp ADC đã scale mV | bsp_adc_monitor.h (line 7) |